# School / Company Network Design Project Report

Design and Configure a Small Company Network Using VLANs

## 1. Introduction

Modern organizations rely heavily on computer networks to support communication, resource sharing, and business operations. In a company environment where multiple departments operate together, proper organization and security are essential. One of the best ways to improve network management and security is through the use of Virtual Local Area Networks (VLANs).network

This project focuses on designing and configuring a small company network using VLANs in Cisco Packet Tracer. The company contains five departments: Administration, IT, Human Resources, Sales, and Reception. Each department is separated into its own VLAN with a unique IP network. The use of VLANs improves network performance, security, and organization by preventing unnecessary communication between departments.

## 2. Project Objectives

To design a small company network topology using switches and PCs.

To create separate VLANs for different departments.

To assign IP addresses to all devices.

To configure switch ports for VLAN communication.

To test connectivity within VLANs.

To improve network security through VLAN separation.

## 3. Network Requirements

Two Cisco 2960 switches

15 PCs

2 printers

An access point

Copper straight-through cables

One crossover cable for switch-to-switch connection

Cisco Packet Tracer software

## 4. Network Topology Design

The network topology was designed using two switches connected together through a trunk link. Each department contains two PCs connected to the switches using access ports. The trunk connection allows VLAN information to pass between switches while maintaining VLAN separation.

Departments included in the topology are:

Admin

IT

HR

Sales

Reception

## 5. VLAN Table

| Department | VLAN ID | VLAN Name |
| --- | --- | --- |
| Admin | 10 | ADMIN |
| IT | 20 | IT |
| HR | 30 | HR |
| Sales | 40 | SALES |
| Reception | 50 | RECEPTION |

## 6. IP Addressing Table

| Department | Network | PC Address Range | Subnet Mask |
| --- | --- | --- | --- |
| Admin | 192.168.1.0/24 | 192.168.1.2 - 192.168.1.5 | 255.255.255.0 |
| IT | 192.168.2.0/24 | 192.168.2.2 - 192.168.2.4 | 255.255.255.0 |
| HR | 192.168.3.0/24 | 192.168.3.2 - 192.168.3.4 | 255.255.255.0 |
| Sales | 192.168.4.0/24 | 192.168.4.2 - 192.168.4.5 | 255.255.255.0 |
| Reception | 192.168.5.0/24 | 192.168.5.2 - 192.168.5.6 | 255.255.255.0 |

## 7. Configuration Commands Used

enable
configure terminal
vlan 10
name ADMIN
vlan 20
name IT
vlan 30
name HR
vlan 40
name SALES
vlan 50
name RECEPTION
interface range fa0/1 - 2
switchport mode access
switchport access vlan 10
switchport port-security

switchport port-security max 1

switshport port-security violation sh

switchport port-security mac-address sticky

## 8. Network Testing and Security

After configuration, several connectivity tests were performed to verify that the network was functioning correctly. Devices within the same VLAN successfully communicated with each other using the ping command. Devices in different VLANs were unable to communicate, confirming that VLAN isolation was successful.

Ping tests between devices in the same VLAN were successful.

Ping tests between devices in different VLANs failed as expected.

The command 'show vlan brief' confirmed proper VLAN assignment.

The command 'show interfaces trunk' verified trunk functionality.

Basic port security configurations were implemented.

## 9. Challenges Faced During Implementation

Understanding VLAN concepts for the first time.

Assigning correct ports to VLANs.

Understanding the security command.

Troubleshooting failed connectivity tests.

Learning Cisco commands.

The reception laptop not able to connect with the reception wifi

## 10. Conclusion

This project successfully demonstrated how VLANs can be used to organize and secure a company network. By separating departments into different VLANs, the network became more secure and easier to manage. The project also improved practical networking skills such as switch configuration, IP addressing, VLAN configuration, trunking, and network testing using Cisco Packet Tracer.

The project provided valuable hands-on experience in network design and helped build a strong foundation in computer networking concepts.